The DVB-RCS MIB
draft-combes-ipdvb-mib-rcs-04.doc

Status of this Memo

By submitting this Internet-Draft, each author represents that       any applicable patent or other IPR claims of which he or she is       aware have been or will be disclosed, and any of which he or she       becomes aware will be disclosed, in accordance with Section 6 of       BCP 79.

This document may not be modified, and derivative works of it may not be created, except to publish it as an RFC and to translate it into languages other than English, or to extract section 3 as-is for separate use.

Internet-Drafts are working documents of the Internet Engineering Task Force (IETF), its areas, and its working groups.  Note that other groups may also distribute working documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six months and may be updated, replaced, or obsoleted by other documents at any time.  It is inappropriate to use Internet-Drafts as reference material or to cite them other than as "work in progress."

The list of current Internet-Drafts can be accessed at
     http://www.ietf.org/ietf/1id-abstracts.txt

The list of Internet-Draft Shadow Directories can be accessed at
     http://www.ietf.org/shadow.html

This Internet-Draft will expire on February 29, 2009.

Abstract

This document describes the MIB module for the Digital Video Broadcasting Return Channel via Satellite system (DVB-RCS). It defines a set of MIB entities to characterize the behavior and performance of network layer entities deploying DVB-RCS.

Table of Contents

1. Introduction	3

2. Conventions used in this document	5

2.1. Abbreviations	5

2.2. Glossary	7

2.2.1. Star DVB-RCS network	7

2.2.2. Mesh DVB-RCS network	7

2.2.3. Transparent DVB-RCS network	8

2.2.4. Regenerative DVB-RCS network	8

2.2.5. DVB-RCS MAC layer	8

2.2.6. DVB-RCS Downlink	8

2.2.7. DVB-RCS Uplink	8

2.2.8. IDU	8

2.2.9. ODU	9

2.2.10. RCST	9

2.2.11. NCC	9

2.2.12. Configuration file	9

2.2.13. Logfile	9

2.2.14. InstallLogfile	9

2.2.15. Antenna alignment	9

2.2.16. CW frequency	9

2.2.17. Request Class	9

2.2.18. Channel ID	10

2.2.19. ATM profile	10

2.2.20. MPEG profile	10

2.2.21. PID pool	10

2.2.22. Capacity Categories	10

2.2.23. Start transponder	11

2.2.24. DVB-S	11

2.2.25. DVB-S2 and CCM/VCM/ACM	12

2.2.26. Interactive Network	12

3. MIB Module overview	12

3.1. Textual Conventions	13

3.2. Structure of the MIB	13

3.3. Relationship to the Interfaces MIB Module	15

3.4. MIB groups description	19

3.4.1. rcstSystem	19

3.4.2. rcstNetwork	19

3.4.3. rcstInstall	19

3.4.4. rcstQos	19

3.4.5. rcstControl	20

3.4.6. rcstState	20

3.4.7. fwdLink (fwdConfig and fwdStatus groups)	20

3.4.8. rtnLink (rtnConfig and rtnStatus groups)	20

4. Definitions	21

5. Security Considerations	86

6. IANA Considerations	87

7. Acknowledgments	87

8. References	88

8.1. Normative References	88

8.2. Informative References	88

9. Authors' Addresses	90

10. Intellectual Property Statement	90

11. Disclaimer of Validity	91

12. Copyright Statement	91

# Introduction

The SatLabs Group [SATLABS] is an international non-profit EEIG (European Economic Interest Grouping) committed to large-scale adoption and deployment of the Digital Video Broadcasting Return Channel via Satellite (DVB-RCS) standard [ETSI-RCS]. SatLabs members are service providers, satellite operators, system integrators, terminal manufacturers and technology providers with an interest in DVB-RCS.

Since its creation in 2001, the main goal of the SatLabs Group has been to achieve interoperability between DVB-RCS terminals and systems. Therefore, the Group has defined the SatLabs Qualification Program that provides an independent certification process for DVB-RCS Terminals based on System Recommendations defined by SatLabs. To enhance products interoperability, beyond the physical and MAC layer mechanisms defined in the DVB-RCS standard, SatLabs has expanded its Recommendations in the field of DVB-RCS terminal management [SATLABS]. As a part of this effort, SatLabs has specified a common SNMP Management Information Base (MIB) for DVB-RCS terminals, which is defined in this document.

A DVB-RCS terminal is denoted as a Return Channel Satellite Terminal (RCST) in the remainder of this document. This consists of an Indoor Unit (IDU) and an Outdoor Unit (ODU) connected through an Inter Facility Link (IFL), usually a coaxial L-band interface. On the user side, the IDU is connected to the user network through a Local Area Network (LAN) interface (usually Ethernet). On the network side, the ODU is connected via a satellite link (the Air Interface).

The DVB-RCS MIB is implemented in the IDU of an RCST. RCST management can be performed either through the LAN interface (Local management) or through the Air Interface (Remote management from the Network Control Center, NCC). RCST and NCC elements are shown on Figure 1.

+------------+

|  IP        |

|  End Host  |

+-----+------+

|

- - - - - - - -|- - - - - - - - - - - - - - - -

|              | LAN interface                 |

|

|       +------+--------+                      |

| Indoor Unit   |

|       |  (IDU)        |                      |

+------+--------+

|              |                               |

Inter Facility Link (IFL)

|              |                               |

+-----+--------+

|       | OutDoor Unit  |                      |

| (ODU)         |

|       +------+--------+                      |

|

|              | Air Interface                 |

- - - - - - -  |- - - - - - - - - - - - - - - -

RCST         |

|        +----------------+

+------->| Network Control|

| Center (NCC)   |

+----------------+

FIGURE 1: RCST architecture

# Conventions used in this document

This memo defines a portion of the Management Information Base (MIB) for use with network management protocols in the Internet community.

For a detailed overview of the documents that describe the current Internet-Standard Management Framework, please refer to section 7 of RFC 3410 [RFC3410].

Managed objects are accessed via a virtual information store, termed the Management Information Base or MIB. MIB objects are generally accessed through the Simple Network Management Protocol (SNMP). Objects in the MIB are defined using the mechanisms defined in the Structure of Management Information (SMI).  This memo specifies a MIB module that is compliant to the SMIv2, which is described in STD 58, RFC 2578 [RFC2578], STD 58, RFC 2579 [RFC2579] and STD 58, RFC 2580 [RFC2580].

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT", "SHOULD", "SHOULD NOT", "RECOMMENDED", "MAY", and "OPTIONAL" in this document are to be interpreted as described in RFC 2119 [RFC2119].

## Abbreviations

AAL5	 ATM Adaptation Layer Type 5

ACM    Adaptive Coding and Modulation (defined in [ETSI-DVBS2])

ATM    Asynchronous Transfer Mode

AVBDC  Absolute Volume-Based Dynamic Capacity

BER    Bit Error Ratio

BUC    Block Up-Converter

CCM    Constant Coding and Modulation

CNR    Carrier to Noise Ratio

CRA    Continuous Rate Assignment

CSC    Common Signalling Channel

CW     Continuous Wave (carrier frequency)

dBi    deciBel (isotropic)

dBm    deciBel (with respect to 1 mW)

DSCP   DiffServ Code Point

ETSI   European Telecommunications Standards Institute

FEC	 Forward Error Correction

GS		 Generic Stream

GSE	 Generic Stream Encapsulation

IDU    InDoor Unit

IFL    Inter-Facility Link

LNB    Low Noise Block

LO     Local Oscillator

MAC    Medium Access Control

MIB    Management Information Base

MPEG   Motion Pictures Expert Group

MPE	 Multi-Protocol Encapsulation

NCC    Network Control Centre

OAM    Operations and Management

ODU    OutDoor Unit

PHB    Per-Hop Behavior

PID    Packet IDentifier (MPEG, used as Program Identifier in DVB)

QoS	 Quality of Service

RBDC   Rate-Based Dynamic Capacity

RC     Request Class

RCST   Return Channel via Satellite Terminal (DVB-RCS Terminal)

Rx     Receive

SDU	 Service Data Unit

SSPA   Solid State Power Amplifier

TDM    Time Division Multiplex

TDMA   Time Division Multiple Access

TS		 Transport Stream (as defined by MPEG)

Tx     Transmit

VBDC   Volume-Based Dynamic Capacity

VCI    Virtual Channel Identifier (ATM)

VPI    Virtual Path Identifier (ATM)

Vpp    Volts peak-to-peak

## Glossary

The terms in this document are derived either from DVB-RCS standard specifications [ETSI-RCS] or from SatLabs System Recommendations [SATLABS].

### Star DVB-RCS network

This denotes a hub-and-spoke configuration where all communications pass through a central hub, that usually also includes the NCC. Peer-to-peer communication between RCSTs is possible, but through a double satellite hop (this traffic has to pass through the hub).

### Mesh DVB-RCS network

This denotes a mesh configuration that supports peer-to-peer communications in a single satellite hop directly between RCSTs.

### Transparent DVB-RCS network

This denotes a network using transparent satellite transponders. Star or mesh network configurations can be supported. In the case of a mesh configuration, RCSTs need to incorporate a TDMA receiver in addition to the TDM receiver.

### Regenerative DVB-RCS network

This denotes a network using regenerative satellite transponders, i.e. including some On-Board Processing functionality allowing demodulation and decoding of the uplink TDMA signals and re-multiplex the traffic on the downlink. Star or mesh network configurations can be supported.

### DVB-RCS MAC layer

The DVB-RCS MAC Layer represents the air interface of an RCST, as defined in the DVB-RCS standard. The interface supports star and mesh networks and is bi-directional.

### DVB-RCS Downlink

The downlink is a DVB-RCS physical link based on TDM. It corresponds to the forward link of a DVB-RCS transparent system or the downlink of a DVB-RCS regenerative system. It is based on either the DVB-S or DVB-S2 standard. In the DVB-RCS context, this interface is bi-directional.

### DVB-RCS Uplink

The DVB-RCS Uplink is based on TDMA. It corresponds to the return or mesh link of a RCS transparent system or the uplink of a RCS regenerative system. It is based on the DVB-RCS standard.

In the context of star transparent and mesh regenerative DVB-RCS systems, this interface is uni-directional.

In the context of mesh transparent DVB-RCS systems, this interface is bi-directional.

### IDU

This is the indoor part of the RCST (including at least the power supply, and usually also the modem and networking functions).

### ODU

This is the outdoor part of the RCST (including at least the aerial, and usually also the LNB and BUC).

### RCST

This is the Satellite Terminal, installed on the customer premises. It is composed of the IDU and ODU.

### NCC

The NCC provides Control and Monitoring Functions. It generates control and timing signals for the operation of the DVB-RCS Network.

### Configuration file

The configuration file is XML formatted file, storing configuration parameters for the RCST and their values.

### Logfile

The logfile is stored at the RCST. This is used to log particular events that occurr on RCST side.

### InstallLogfile

The Installlogfile is stored at the RCST. This logs particular events that occur on RCST side, related to RCST installation phase.

### Antenna alignment

This is the process to align the RCST antenna, part of the ODU, in order to enable bi-directional communication (uplink, downlink) with the satellite network.

### CW frequency

The CW frequency is the frequency of a Continuous Wave signal. It is a narrowband carrier transmitted for the duration of measurements during the installation of a RCST.

### Request Class

A Request Class (RC) is a representation of a Per Hop Behavior (PHB) at the MAC layer. It defines a behavior of the MAC layer for a given aggregation of traffic. This behavior includes a combination of Capacity Categories associated to the RC and a Priority with respect to the other RCs supported by a RCST.

### Channel ID

Each Request Class is identified by a unique Channel_ID in the communication between the RCST and the NCC.

### ATM profile

The ATM profile is one of the two profiles for traffic burst format on a DVB-RCS uplink. It is based on one or more concatenated ATM cells, each of length 53 bytes, plus an optional prefix.

### MPEG profile

The MPEG profile is one of the two profiles for traffic burst format on the DVB-RCS uplink. It is based on a number of concatenated MPEG2-TS packets, each of length 188 bytes.

### PID pool

For the MPEG profile several RCs may be mapped within a pool of several PIDs to allow cross-RC Section Packing [RFC4259]. Section packing can be used on all PIDs and higher priority traffic can always pre-empt lower priority streams. This reduces the need for padding.

### Capacity Categories

The TDMA timeslot allocation process for the DVB-RCS uplink supports several capacity categories.

The Capacity Categories CRA, RBDC and A/VBDC, when authorized for a RC, have to be configured from the NCC. These configuration parameters are used to inform the RCST of the configuration of each Category at the NCC side and thus help in Capacity Requests computation.

The configuration of these parameters is performed for each RC. A SatLabs optional feature is defined that allows their configuration at the RCST level in addition to configuration per RC. This feature is denoted RCST_PARA.

#### Continuous Rate Assignment (CRA)

CRA is a rate capacity that is provided in full in a continuous manner to the RCST while required.

#### Rate-Based Dynamic Capacity (RBDC)

RBDC is rate capacity that is requested dynamically by an RCST. RBDC capacity is provided in response to explicit requests from the RCST to the NCC, such requests being absolute (i.e. corresponding to the full rate currently being requested). Each request overrides all previous RBDC requests from the same RCST, and is subject to a maximum rate limit.

#### Volume-Based Dynamic Capacity (VBDC)

VBDC is volume capacity that is requested dynamically by an RCST. VBDC capacity is provided in response to explicit requests from the RCST to the NCC, such requests being cumulative (i.e. each request adds to all previous requests from the same RCST).

#### Absolute Volume-Based Dynamic Capacity (AVBDC)

AVBDC is volume capacity that is requested dynamically by an RCST. This capacity is provided in response to explicit requests from the RCST to the NCC, such requests being absolute (i.e. this request replaces the previous ones from the same RCST).

The combination of AVBDC and VBDC is seen as a single Capacity Category, denoted A/VBDC.

#### Population ID

This defines a group of RCSTs within a Network.

### Start transponder

This is the satellite transponder on which the communication is initiated from a RCST point-of-view when in the installation mode. The parameters corresponding to this transponder (satellite orbital position, frequency, etc.) are stored at the RCST as power-up configuration data.

### DVB-S

DVB-S is the Digital Video Broadcast over Satellite [ETSI-DVBS]. It is a framework and set of associated standards published by ETSI for the transmission of video, audio, and data, using the ISO MPEG-2 Standard [ISO-MPEG], over satellite links.

### DVB-S2 and CCM/VCM/ACM

DVB-S2 is the Second Generation of the Digital Video Broadcast for Satellite applications standard [ETSI-DVBS2]. It is a framework and set of associated standards published by ETSI for the transmission of video, audio and data.

BBFRAME: The main framing unit of the DVB-S2 protocol stack.

CCM: In CCM transmission mode, the forward link uses a constant set of transmission parameters (FEC coding rate and modulation scheme) for all receivers.

VCM: In VCM transmission mode, the forward link uses transmission parameters which are variable on a BBFRAME-by-BBFRAME, but fixed on a Receiver basis, according to fixed link and propagation conditions for each Receiver.

ACM: In ACM transmission mode, the forward link uses transmission parameters which are dynamically adjusted on a BBFRAME-by-BBFRAME and Receiver-per-Receiver basis, according to actual link and propagation conditions.  In order to implement ACM, feedback from each Receiver has to be provided by DVB-RCS return channel.

### Interactive Network

This is another name for a DVB-RCS based satellite network.

# MIB Module overview

This MIB module provides a set of objects required for the management of SatLabs-compliant RCST.  The specification is derived from the parameters and protocols described in [SATLABS].

The MIB module in this document uses the following OBJECT IDENTIFIER values as already assigned by IANA under the smi-numbers registry [IANA]:

+------------+---------------------------+

| Descriptor | OBJECT IDENTIFIER value   |

+------------+---------------------------+

|dvbrcsMib   |{ mib-2 transmission 239 } |

+------------+---------------------------+

Table 1: Object Identifiers for the MIB

These values have been assigned for this MIB under the 'mib-2.transmission' subtree.

## Textual Conventions

This MIB module defines new textual conventions for RCST indications of SatLabs defined capabilities, including profiles, options and optional features.

systemSatLabsProfileMap represents the SatLabs profiles supported as defined in [SATLABS].

systemSatLabsOptionMap represents the SatLabs options supported as defined in [SATLABS]. These are options that are used for the certification of SatLabs terminals. They represent important functionality, with impact on interoperability, and their support is advertised with the RCST certification level.

systemSatLabsFeatureMap represents the SatLabs optional features supported as defined in [SATLABS]. These represent minor features, not necessary for interoperability. They are not used for the certification of SatLabs terminals.

## Structure of the MIB

This MIB module is structured into two top level groups:

- The dvbrcsMIBobjects group includes all the managed objects of the DVB-RCS MIB.

- The dvbrcsConformance group includes the compliance statements for DVB-RCS terminals that are compliant with [SATLABS]. The managed objects are grouped into formal object groups (i.e. units of conformance) according to the relation to specific SatLabs options or features. The conformance statements (MODULE-COMPLIANCE specification) are described within the rcstCompliance group while the units of conformance are described within the rcstGroups group.

The dvbrcsMIBobjects group is further structured into three groups: rcst, fwdLink and rtnLink.

- The rcst group covers management related to the RCST equipment. It is structured into six groups:

- rcstSystem

- rcstNetwork

- rcstInstall

- rcstQos

- rcstControl

- rcstState

- The fwdLink group covers management information related to the RCST forward link. It is structured into two groups:

- fwdConfig

- fwdStatus

- The rtnLink group covers management information related to the RCST return link. It is structured into two groups:

- rtnConfig

- rtnStatus

Tables within each of these groups cover different functions like return link traffic management (packet classes, Request Classes, PID pools) and forward links configuration and status.

Rows created automatically (e.g., by the device according to the hardware configuration) may and generally will have a mixture of configuration and status objects within them.  Rows that are meant to be created by the management station are generally restricted to configuration (read-create) objects.

The MIB module finally includes a conformance section including conformance groups and compliance statements.

## Relationship to the Interfaces MIB Module

This section clarifies the relationship of this MIB module to the Interfaces MIB [RFC2863]. Several areas of correlation are addressed in the following. The implementer is referred to the Interfaces MIB document in order to understand the general intent of these areas.

IANA has assigned three ifType labels for DVB-RCS. Each RCST MUST support at least the three following interfaces:

- dvbRcsMacLayer (239), -- DVB-RCS MAC Layer

- DVB-RCS MAC Layer represents the air interface of a RCST, as defined in [ETSI-RCS]. This interface supports star and mesh networks and is bi-directional. Only star networks are considered by the present MIB module.

- dvbTdm (240), -- DVB Satellite TDM

- DVB-RCS Physical link based on Time Division Multiplexing. It corresponds to the forward link of a RCS transparent system or the downlink of a RCS regenerative system. It is based on either DVB-S or DVB-S2 standard. Only transparent systems are considered by the present MIB module.

In the DVB-RCS context, this interface is uni-directional (i.e. forward link, or downstream).

- dvbRcsTdma (241), -- DVB-RCS TDMA

DVB-RCS Physical link based on Time Division Multiple Access. It corresponds to the return or mesh link of a RCS transparent system or the uplink of a RCS regenerative system. It is based on the DVB-RCS standard.

In the context of star transparent and mesh regenerative DVB-RCS systems, this interface is uni-directional.

In the context of mesh transparent DVB-RCS systems, this interface is bi-directional.

- Only star transparent systems are considered by the present MIB module (i.e. return link, or upstream).

The protocol stack (as reflected in ifStackTable) will be as follows:

+--------------------------+

|            IP            |

+---------------+----------+

+  dvbRcsTdma   + dvbTdm   +

+---------------+----------+

+   MPEG/ATM    + MPEG/GS  +

+---------------+----------+

FIGURE 2: RCST protocol stack

GS (Generic Stream) is defined in [ETSI-DVBS2].

The additional Ethernet interface is used on the LAN side of the RCST.

An instance of ifEntry exists for each DvbTdm interface, for each DvbRcsTdma (normally only one), and for each DvbRcsMac layer (normally only one).

The interface counters relate to:

- dvbRcsMacLayer: DVB-RCS two-way MAC that counts aggregate DVB-RCS SDU traffic (IP layer)

- dvbTdm: DVB-RCS TDM that counts either Generic Stream Encapsulation (GSE) or Multi-Protocol Encapsulation (MPE) traffic, as applicable for the specific configuration

GSE is defined in [ETSI-GSE]

MPE is defined in [ETSI-DAT]

- dvbRcsTdma: DVB-RCS TDMA that counts either ATM Adaptation Layer 5 (AAL5) or MPE traffic, as applicable for the specific configuration

The ifStackTable [RFC2863] MUST be implemented to identify the relationships among sub-interfaces.

The following example is a DVB-RCS star network with DVB-S and DVB-RCS. As illustrated on Figure 3, it shows a DVB-RCS MAC interface with one downstream and one upstream interfaces. In this network, ATM encapsulation is used in the DVB-RCS uplink. Two ATM Logical Ports are shown. DVB-S2 or DVB-S can be used in the downlink. MPEG can be used as well instead of ATM Logical Ports.

+----------------------------------------------------------+

|                 IP Network Layer                         |

+------+----------------------------------+----------------+

|                                  |

+------+-------+       +------------------+----------------+

| Ethernet LAN |       |          dvbRcsMacLayer           |

+--------------+       +-------------+---------------------+

|                 |

+-------------+-----------+ +---+---+

|        dvbRcsTdma       | |dvbTdm |

+-----+-------------+-----+ +-------+

|             |

+-----+-----+ +-----+-----+

|atm-logical| |atm-logical|

+-----------+ +-----------+

FIGURE 3: Example stacking

As can be seen from this example, the dvbRcsMacLayer interface is layered on top of the downstream and upstream interfaces, and the upstream interface is layered on top of upstream ATM logical links.

In this example, the assignment of index values could be as follows:

ifIndex       ifType                     Description

2    dvbRcsMacLayer (239)     DVB-RCS MAC Layer

3    dvbRcsTdma (240)         DVB-RCS TDMA Upstream

4    dvbTdm(241)              DVB-RCS TDM Downstream

5    atm-logical(80)          ATM Logical Port

6    atm-logical(80)          ATM Logical Port

The corresponding ifStack entries would then be:

+--------------------+-------------------+

| IfStackHigherLayer | ifStackLowerLayer |

+--------------------+-------------------+

|         0          |         1         |

|         0          |         2         |

|         1          |         0         |

|         2          |         3         |

|         2          |         4         |

|         3          |         5         |

|         3          |         6         |

|         4          |         0         |

|         5          |         0         |

|         6          |         0         |

+--------------------+-------------------+

FIGURE 1: Example ifStack entries

## MIB groups description

### rcstSystem

The MIB objects in this group gather some basic information that would allow anyone to trace the history - the life - of the RCST as well as to get a complete description of its constitution on the component point of view, including the SatLabs options/features support statement. Many of the parameters will be defined at installation.

This group contains description parameters related to the RCST type (ODU type) and location. These parameters are believed to stay unchanged once it has been defined during installation. Modification of hardware equipment, maintenance operations and geographical re-location may require an update of those MIB objects. They are. Note that rcstSystem.systemLocation object gives the location of the ODU antenna, which is needed for network operation, while the system.sysLocation (MIB-II SNMP OID) provides the location of the IDU unit, which can not be used for the same purpose.

### rcstNetwork

This group contains all the MIB objects related to network parameters.

In this subgroup, two objects have been defined in order to differentiate between control and user traffic and associate them with a physical interface. Both rcstNetwork.networkLanIpAddress (Traffic) and rcstNetwork.networkOamIpAddress (OAM) provide the value of the IP address of, respectively, the user traffic and the control and management traffic.

### rcstInstall

This group contains all the information related to the RCST installation and commissioning. Many parameters are believed to stay unchanged once it has been defined during installation. Modification of hardware equipment, maintenance operations and geographical re-location may require an update of those MIB objects.

### rcstQos

This group contains objects to configure the Quality of Service (QoS) of the RCST by the NCC.

The pktClass table defines the packet classification for IP layer 3 classifications. Each pktClass entry is mapped to a phbEntry in the phbMappingTable.

The phbMappingTable makes the relation between a packet classification entry, a Per-Hop Behavior (PHB) identifier and a Request class entry.

The requestClassTable defines all the layer 2 DVB-RCS QoS parameters.

### rcstControl

This MIB group contains objects a network manager can use to invoke actions and tests supported by the RCST agent and to retrieve the action/test results.

### rcstState

This MIB group describes the fault state, software versions and configuration file versions of the RCST.

### fwdLink (fwdConfig and fwdStatus groups)

This MIB group contains parameters that enable the NCC to have access to data about the forward link.

Configuration information is kept into the fwdLink.fwdConfig subgroup. Status information is kept into the fwdLink.fwdStatus subgroup.

The information of the fwdLink.fwdConfig.fwdStartTable are used for the first time the RCST tries to acquire the forward link. All these objects values are aligned with the Satellite Delivery System Descriptor in the Network Information Table (NIT) table [ETSI-SI].

The objects in the fwdLink.fwdConfig.fwdStatusTable are aligned with the satellite forward path descriptor form the RCS Map Table (RMT) [ETSI-RCS] and with the Physical Layer (PL) Header [ETSI-DVBS2], which specified the MODCOD (modulation and FEC rate) and the Type (frame length short of long and the presence/absence of pilots).

### rtnLink (rtnConfig and rtnStatus groups)

This MIB group contains parameters that enable the NCC to have access to data about the return link.

Configuration information is kept into the rtnLink.rtnConfig subgroup. Status information is kept into the rtnLink.rtnStatus subgroup.

The RCST is only able to deal with one return link at a time. Hence, there is no need to define a table to collect the different SNMP objects, as it is done for the forward.

# Definitions

DVBRCS-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY,

Integer32,

IpAddress,

transmission,

OBJECT-TYPE

FROM SNMPv2-SMI        -- [RFC2578]

TEXTUAL-CONVENTION,

DisplayString,

RowStatus

FROM SNMPv2-TC         -- [RFC2579]

OBJECT-GROUP,

MODULE-COMPLIANCE

FROM SNMPv2-CONF;      -- [RFC2580]

dvbrcsMIB MODULE-IDENTITY

LAST-UPDATED "200807291200Z"

ORGANIZATION "The SatLabs Group"

CONTACT-INFO

"The SatLabs Group

Web:    www.satlabs.org

E-mail: info@satlabs.org"

DESCRIPTION

"DVB-RCS MIB subtree.

This MIB module applies to equipment that is a

Return Channel Satellite Terminal (RCST) defined in

the Digital Video Broadcasting Return

Channel via Satellite system (DVB-RCS) standard (ETSI EN 301

791 Digital Video Broadcasting (DVB); Interaction Channel

for Satellite Distribution Systems, European

Telecommunications Standards Institute (ETSI)).

It defines a set of MIB entities to characterise the

behaviour and performance of network layer entities

implementing DVB-RCS.

This MIB module is intended to be used by DVB-RCS equipment

following the SatLabs System Recommendations, defined by the

SatLabs Group and available at www.satlabs.org."

REVISION "200807291200Z"

DESCRIPTION

"Revision of this MIB module, following comments from ipdvb

WG."

::= { transmission 239 }

===================================================================

-- Textual Conventions

--===================================================================

SatLabsProfileMap ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"This textual convention enumerates the declaration of the

SatLabs defined terminal profiles. (0) refers to the most

significant bit."

REFERENCE

"SatLabs System Recommendations available at www.satlabs.org"

SYNTAX BITS     {

dvbs(0),

dvbs2ccm(1),

dvbs2acm(2),

spare1(3),

spare2(4),

spare3(5),

spare4(6),

spare5(7),

spare6(8),

spare7(9),

spare8(10),

spare9(11),

spare10(12),

spare11(13),

spare12(14),

spare13(15),

spare14(16),

spare15(17),

spare16(18),

spare17(19),

spare18(20),

spare19(21),

spare20(22),

spare21(23),

spare22(24),

spare23(25),

spare24(26),

spare25(27),

spare26(28),

spare27(29),

spare28(30),

spare29(31)

}

SatLabsOptionMap ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"This textual convention enumerates the declaration of the

SatLabs defined options. (0) refers to the most significant

bit."

REFERENCE

"SatLabs System Recommendations available at www.satlabs.org"

SYNTAX BITS     {

mpegTrf(0),

coarseSync(1),

wideHop(2),

fastHop(3),

dynamicMfTdma(4),

contentionSync(5),

qpskLow(6),

mod16Apsk(7),

mod32Apsk(8),

normalFec(9),

multiTs(10),

gsTs(11),

enhQoS(12),

pep(13),

http(14),

ftp(15),

dns(16),

chIdStrict(17),

nlid(18),

snmpMisc(19),

spare1(20),

spare2(21),

spare3(22),

spare4(23),

spare5(24),

spare6(25),

spare7(26),

spare8(27),

spare9(28),

spare10(29),

spare11(30),

spare12(31)

}

SatLabsFeatureMap ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"This textual convention enumerates the declaration of

the SatLabs specified compapatibility and configuration

features. (0) refers to the most significant bit."

REFERENCE

"SatLabs System Recommendations available at

www.satlabs.org"

SYNTAX BITS     {

rcstPara(0),

installLog(1),

enhClassifier(2),

routeId(3),

spare1(4),

spare2(5),

spare3(6),

spare4(7),

spare5(8),

spare6(9),

spare7(10),

spare8(11),

spare9(12),

spare10(13),

spare11(14),

spare12(15),

spare13(16),

spare14(17),

spare15(18),

spare16(19),

spare17(20),

spare18(21),

spare19(22),

spare20(23),

spare21(24),

spare22(25),

spare23(26),

spare24(27),

spare25(28),

spare26(29),

spare27(30),

spare28(31)

}

--===================================================================

-- object type definitions

--===================================================================

dvbrcsMIBObjects      OBJECT IDENTIFIER ::= {dvbrcsMIB 1}

dvbrcsConformance     OBJECT IDENTIFIER ::= {dvbrcsMIB 2}

rcst        OBJECT IDENTIFIER ::= {dvbrcsMIBObjects 1}

fwdLink     OBJECT IDENTIFIER ::= {dvbrcsMIBObjects 2}

rtnLink     OBJECT IDENTIFIER ::= {dvbrcsMIBObjects 3}

rcstSystem        OBJECT IDENTIFIER ::= {rcst 1}

rcstNetwork       OBJECT IDENTIFIER ::= {rcst 2}

rcstInstall       OBJECT IDENTIFIER ::= {rcst 3}

rcstQos           OBJECT IDENTIFIER ::= {rcst 4}

rcstControl       OBJECT IDENTIFIER ::= {rcst 5}

rcstState         OBJECT IDENTIFIER ::= {rcst 6}

fwdConfig         OBJECT IDENTIFIER ::= {fwdLink 1}

fwdStatus         OBJECT IDENTIFIER ::= {fwdLink 2}

rtnConfig         OBJECT IDENTIFIER ::= {rtnLink 1}

rtnStatus         OBJECT IDENTIFIER ::= {rtnLink 2}

--===================================================================

---    rcstSystem sub-tree object types

--===================================================================

systemMibRevision        OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"This object allows the SNMP agent to report the implemented

MIB module revision.

The supported REVISION of this module is reported."

::= {rcstSystem 1}

--===================================================================

-- Options declared according to the textual conventions

--===================================================================

systemSatLabsProfilesDeclaration OBJECT-TYPE

SYNTAX        SatLabsProfileMap

MAX-ACCESS    read-only

STATUS        current

DESCRIPTION

"Indicates the SatLabs profiles supported as defined in the

SatLabs System Recommendations. A value of 1 indicates that

the respective option is supported. The mapping to the

profiles is to be understood as described here. (0) refers

to the most significant bit.

dvbs(0) -> DVBS profile (DVB-S support)

dvbs2ccm(1) -> DVB-S2 CCM profile (CCM support)

dvbs2acm(2) -> DVB-S2 ACM profile (CCM, VCM and ACM

support)"

REFERENCE

"SatLabs System Recommendations available at www.satlabs.org"

::= {rcstSystem 2}

systemSatLabsOptionsDeclaration OBJECT-TYPE

SYNTAX        SatLabsOptionMap

MAX-ACCESS    read-only

STATUS        current

DESCRIPTION

"Indicates the SatLabs options supported as defined in the

SatLabs System Recommendations. A value of 1 indicates that

the respective option is supported. 	The mapping to the

options is to be understood as described 	here.(0) refers to

the most significant bit.

mpegTrf(0) -> MPEG_TRF

coarseSync(1) -> COARSE_SYNC

wideHop(2) -> WIDE_HOPP

fastHop(3) -> FAST_HOPP

dynamicMfTdma(4) -> Dynamic_MF_TDMA

contentionSync(5) -> CONTENTION_SYNC

qpskLow(6) -> QPSKLOW

mod16Apsk(7) -> 16APSK

mod32Apsk(8) -> 32APSK

normalFec(9) -> NORMALFEC

multiTs(10) -> MULTITS

gsTs(11) -> GSTS

enhQoS(12) -> ENHQOS

pep(13) -> PEP

http(14) -> HTTP

ftp(15) -> FTP

dns(16) -> DNS

chIdStrict(17) -> CHID_STRICT

nlid(18) -> NLID

snmpMisc(19) -> SNMPMISC

The support of specific options mandates the support of

specific objects and access levels."

REFERENCE

"SatLabs System Recommendation available at www.satlabs.org"

::= {rcstSystem 3}

systemSatLabsFeaturesDeclaration OBJECT-TYPE

SYNTAX        SatLabsFeatureMap

MAX-ACCESS    read-only

STATUS        current

DESCRIPTION

"Indicates the optional compatibility features and minor

options supported as defined in the SatLabs System

Recommendations. A value of 1 indicates that the respective

feature is supported. The mapping to the features is to be

understood as described here. (0) refers to the most

significant bit.

rcstPara(0) -> RCST_PARA feature

installLog(1) -> INSTALL_LOG feature

enhClassifier(2) -> ENHCLASSIFIER feature

routeId(3) -> ROUTE_ID feature

The support of specific features mandates the support of

specific objects and access levels."

REFERENCE

"SatLabs System Recommendation available at www.satlabs.org"

::= {rcstSystem 4}

systemLocation OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Physical location of the ODU antenna expressed as

Longitude, Latitude and Altitude. The string

shall have 31 characters in the following format:

<xxxx.xxx>,<a>,<yyyyy.yyy>,<b>,<zzzz.z>,M

where x, y and z represents digits,

a=N or S,

b=E or W,

Reading the digits from left to right:

'x' 7 latitude digits; x digits 1-2 contain the degrees,

x digits 3-7 contain the minutes in decimal;

'y' 8 longitude digits; y digits 1-3 contain the degrees,

y digits 4-8 contain the minutes in decimal;

''' 5 altitude digits; meters above sea level in decimal;

'.' is the decimal point;

',' is the field separator;

'M' is the indicator for altitude meters.

This format is a modified subset of the NMEA 0183

(National Marine Electronics Association, Interface

Standard) format for Global Positioning System Fix Data.

This location and the satellite position are used to

calculate the RCST-satellite path delay.

Note: The system.sysLocation object of MIB-II provides physical

location of the IDU unit."

::= {rcstSystem 5}

systemOduAntennaSize OBJECT-TYPE

SYNTAX              DisplayString

UNITS               "cm"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This object gives the diameter of the antenna. This value

shall be given in centimeters. It is defined at

installation.

The object can be used in conjunction with environmental

requirements. "

::= {rcstSystem 6}

systemOduAntennaGain OBJECT-TYPE

SYNTAX              Integer32

UNITS               "x0.1 dBi"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This field describes the antenna peak gain of the ODU and

shall be defined by the installer. The gain shall be given

in tenth of dBi for more flexibility, i.e. 46.5 dBi will be

represented by 465. Defined at installation."

::= {rcstSystem 7}

systemOduSspa OBJECT-TYPE

SYNTAX              Integer32

UNITS               "x0.1 W"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This field describes the Solid State Power Amplifier

installed in the ODU and shall be defined by the installer.

The power shall be given in tenth of a Watt, i.e. 0,5 W will

be represented by 5, 1 W by 10 and 2 W by 20.

Defined at installation."

::= {rcstSystem 8}

systemOduTxType OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Describes the type of transmitter installed in the ODU."

::= {rcstSystem 9}

systemOduRxType OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Describes the type of LNB installed in the ODU, with

information such as vendor type, output type (single, twin,

quad,...), etc."

::= {rcstSystem 10}

systemOduRxBand OBJECT-TYPE

SYNTAX              INTEGER   {

oduHighRxBand (0),

oduLowRxBand  (1)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Describes whether High Band or Low Band is selected in

the LNB. Specifying High Band results in activation of a

18-26 kHz tone with 0.4-0.8 Vpp in the Rx IFL cable:

(0)    - High Band

(1)    - Low Band"

::= {rcstSystem 11}

systemOduRxLO OBJECT-TYPE

SYNTAX              Integer32

UNITS               "x100 Hz"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Frequency of LNB Local Oscillator (in 100 Hz)"

::= {rcstSystem 12}

systemOduTxLO OBJECT-TYPE

SYNTAX              Integer32

UNITS               "x100 Hz"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Frequency of Block Up-Converter Local Oscillator

(in 100 Hz)."

::= {rcstSystem 13}

systemIduPep OBJECT IDENTIFIER ::= {rcstSystem 14}

tcpPep OBJECT-TYPE

SYNTAX	           INTEGER{

disabled (0),

enabled (1)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Status and control of embedded TCP PEP.

0 - disabled or not implemented

1 - enabled                      "

::={ systemIduPep 1}

httpPep	OBJECT-TYPE

SYNTAX              INTEGER{

disabled (0),

enabled (1)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Status and control of embedded HTTP PEP.

0 - disabled or not implemented

1 - enabled                      "

::={ systemIduPep 2}

--===================================================================

-- rcstNetwork sub-tree object types

--===================================================================

networkOamIpAddress OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"OAM IP Address of the RCST. This object is used with both

IP and interfaces MIB-II subgroups. It uniquely determines

the interface through which OAM traffic passes.

The OAM IP address may be statically or dynamically

assigned. It is system dependent whether the OAM IP address

and the Traffic IP address are the same address."

::= {rcstNetwork 1}

networkOamIpNetworkMask OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Network Mask for the OAM IP Address."

::= {rcstNetwork 2}

networkOamIpAddressAssign OBJECT-TYPE

SYNTAX             INTEGER    {

oamIpAddressStatic    (1),

oamIpAddressDynamic    (2)

}

MAX-ACCESS         read-write

STATUS             current

DESCRIPTION

"Identifies whether the OAM IP address is statically

(1) or dynamically (2) assigned."

::= {rcstNetwork 3}

networkLanIpAddress OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"IP address of the LAN interface"

::= {rcstNetwork 4}

networkLanIpNetworkMask OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Mask for the LAN interface"

::= {rcstNetwork 5}

networkAirInterfaceDefaultGateway OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Default Gateway for the air interface"

::= {rcstNetwork 6}

networkDnsServers OBJECT IDENTIFIER ::= {rcstNetwork 7}

primaryDnsServerIpAddress OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"IP address of the primary DNS server in the NCC."

::= { networkDnsServers 1}

secondaryDnsServerIpAddress OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"IP address of the secondary DNS server in the NCC."

::= { networkDnsServers 2}

networkNccMgtIpAddress OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"IP address of the management server in the NCC."

::= {rcstNetwork 8}

networkConfigFileDownloadUrl OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Full path name for the configuration file download.

It includes the protocol type (tftp or ftp) and the

associated server IP address or hostname. Hostname can

only be used if DNS is supported by the RCST.

The format of this parameter follows RFC 1738."

::= {rcstNetwork 9}

networkInstallLogFileDownloadUrl OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Full path of the installation log file to download.

It includes the protocol type (tftp or ftp) and the

associated server IP address or hostname. Hostname can

only be used if DNS is supported by the RCST. The

installation log file can be created on the installer's

computer and downloaded to the RCST.

The format of this parameter follows RFC 1738."

::= {rcstNetwork 10}

networkConfigFileUploadUrl OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Full path name for the configuration file upload.

It includes the protocol type (tftp or ftp) and the

associated server IP address or hostname. Hostname can

only be used if DNS is supported by the RCST.

The format of this parameter follows RFC 1738."

::= {rcstNetwork 11}

networkLogFileUploadUrl OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Full path of the event log file. It includes the protocol

type (tftp or ftp) and the associated server IP address

or hostname. Hostname can only be used if DNS is supported

by the RCST.

The format of this parameter follows RFC 1738."

::= {rcstNetwork 12}

networkInstallLogFileUploadUrl OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Full path of the installation log file. It includes the

protocol type (tftp or ftp) and the associated server

IP address or hostname. Hostname can only be used if DNS is

supported by the RCST. The installation log file can be

retrieved from the RCST by the NCC or by the installer

via the LAN.

The format of this parameter follows RFC 1738."

::= {rcstNetwork 13}

--===================================================================

--    rcstInstall sub-tree object types

--===================================================================

installAntennaAlignmentState OBJECT-TYPE

SYNTAX              INTEGER {

antennaAlignmentStart   (1),

antennaAlignmentDeny    (2),

antennaAlignmentContinue(3),

antennaAlignmentStop    (4),

antennaAlignmentSuccess (5),

antennaAlignmentFail    (6)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Indicates the alignment state of the antenna:

(1)-Start;

(2)-Deny;

(3)-Continue;

(4)-Stop;

(5)-Success;

(6)-Fail"

::= {rcstInstall 1}

installCwFrequency OBJECT-TYPE

SYNTAX              Integer32

UNITS               "x100 Hz"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Frequency at which the transmitted Continuous Wave carrier

(in 100 Hz).

Minimum required precision is 1 kHz."

::= {rcstInstall 2}

installCwMaxDuration OBJECT-TYPE

SYNTAX              Integer32

UNITS        			"seconds"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Time after which the Continuous Wave carrier must be put

down (in seconds)"

::= {rcstInstall 3}

installCwPower OBJECT-TYPE

SYNTAX              Integer32

UNITS               "x0.1 dBm"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"IDU TX output level when the IDU is configured to send CW.

The resolution is 0.1 dBm and the accuracy is +/- 1 dBm.

Reconfiguration is applied immediately to a CW."

::= {rcstInstall 4}

installCoPolReading OBJECT-TYPE

SYNTAX              Integer32

UNITS               "x0.1 dB"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Co-Polarisation measured value during installation

procedure (in 0.1 dB)"

::= {rcstInstall 5}

installXPolReading OBJECT-TYPE

SYNTAX              Integer32

UNITS               "x0.1 dB"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Cross-Polarisation measured value during installation

procedure (in 0.1 dB)"

::= {rcstInstall 6}

installCoPolTarget OBJECT-TYPE

SYNTAX              Integer32

UNITS               "x0.1 dB"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Co-Polarisation target value during installation procedure

(in 0.1 dB)."

::= {rcstInstall 7}

installXPolTarget OBJECT-TYPE

SYNTAX              Integer32

UNITS               "x0.1 dB"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Cross-Polarisation target value during installation procedure

(in 0.1 dB)"

::= {rcstInstall 8}

installStandByDuration OBJECT-TYPE

SYNTAX              Integer32

UNITS               "seconds"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Time to wait in stand-by mode (in seconds)"

::= {rcstInstall 9}

installTargetEsN0 OBJECT-TYPE

SYNTAX              Integer32(0..315)

UNITS               "x0.1 dB"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This value describes the wanted Es/N0 value that enables

operation of the return link with the required error

performance. The values shall be given in tenth of dB and

the initial value shall be equal to 7 dB. The range shall be

from 0 dBm to 31.5 dBm with a precision of 0.1 dB."

DEFVAL        { 70 }

::= {rcstInstall 10}

===================================================================

-- rcstQos sub-tree object types

--===================================================================

pktClassTable OBJECT-TYPE

SYNTAX              SEQUENCE OF PktClassEntry

MAX-ACCESS          not-accessible

STATUS              current

DESCRIPTION

"This table describes the packet classification used in the

DVB-RCS terminal. The number of entries is specified by

PktClassIndex. "

::={rcstQos 1}

pktClassEntry OBJECT-TYPE

SYNTAX            PktClassEntry

MAX-ACCESS        not-accessible

STATUS            current

DESCRIPTION

"An entry in the packet classification table. One object type

of each entry may have a value in the active range (a non-

default value).The other object types are then assumed set to

'inactive'. The entry with the lowest index value takes

precedence when classifying a packet."

INDEX   { pktClassIndex }

::= { pktClassTable 1 }

PktClassEntry ::= SEQUENCE {

pktClassIndex           Integer32,

pktClassDscpLow         Integer32,

pktClassDscpHigh        Integer32,

pktClassDscpMarkValue   Integer32,

pktClassIpProtocol      Integer32,

pktClassIpSrcAddress       IpAddress,

pktClassIpSrcAddressMask   IpAddress,

pktClassIpDstAddress       IpAddress,

pktClassIpDstAddressMask   IpAddress,

pktClassSrcPortLow      Integer32,

pktClassSrcPortHigh     Integer32,

pktClassDstPortLow      Integer32,

pktClassDstPortHigh     Integer32,

pktClassVlanUserPri     Integer32,

pktClassPhbAssociation  Integer32,

pktClassRowStatus       RowStatus

}

pktClassIndex OBJECT-TYPE

SYNTAX              Integer32 (1..64)

MAX-ACCESS          not-accessible

STATUS              current

DESCRIPTION

"Index automatically incremented by one at row creation."

::={pktClassEntry 1}

pktClassDscpLow OBJECT-TYPE

SYNTAX              Integer32 (0..63)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object specifies the low value of a range of DiffServ

Code Point (DSCP) values to which a packet is compared.

A value of 0 is used to inactivate."

DEFVAL        { 0 }

::={pktClassEntry 2}

pktClassDscpHigh OBJECT-TYPE

SYNTAX              Integer32 (0..63)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object specifies the high value of a range of DiffServ

Code Point (DSCP) values to which a packet is compared.

A value of 63 is used to inactivate."

DEFVAL        { 63 }

::={pktClassEntry 3}

pktClassDscpMarkValue OBJECT-TYPE

SYNTAX              Integer32 (-1..63)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object is the DiffServ Code Point (DSCP) value used to

mark the packet, -1 indicates no DSCP marking. Possible DSCP

marks values are (0..63)"

DEFVAL        { -1 }

::={pktClassEntry 4}

pktClassIpProtocol OBJECT-TYPE

SYNTAX              Integer32 (-1..255)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object specifies the IP protocol to which a

packet is compared. A value of 255 indicates inactive."

DEFVAL        { -1 }

::={pktClassEntry 5}

pktClassIpSrcAddress OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object specifies the IP source address to which a

packet is compared."

::={pktClassEntry 6}

pktClassIpSrcAddrMask OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object specifies which bits of the IP source address

will be matched. A mask of all zeros indicates that the

selectivity is inactive."

DEFVAL        { '00000000'H }

::={pktClassEntry 7}

pktClassIpDstAddress OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object specifies the IP destination address to which a

packet is compared."

::={pktClassEntry 8}

pktClassIpDstAddressMask OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object specifies which bits of the IP destination

address will be matched. A mask of all zeros indicates that

the selectivity is inactive."

DEFVAL        { '00000000'H }

::={pktClassEntry 9}

pktClassSrcPortLow OBJECT-TYPE

SYNTAX              Integer32 (-1..65535)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object specifies the low range of the source

port to which a packet is compared. A value of -1 indicates

that the selectivity is inactive."

DEFVAL        { -1 }

::={pktClassEntry 10}

pktClassSrcPortHigh OBJECT-TYPE

SYNTAX              Integer32 (-1..65535)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object specifies the high range of the source port to

which a packet is compared. A value of -1 indicates that the

selectivity is inactive."

DEFVAL        { -1 }

::={pktClassEntry 11}

pktClassDstPortLow OBJECT-TYPE

SYNTAX              Integer32 (-1..65535)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object specifies the low range of the destination

port to which a packet is compared. A value of -1 indicates

that the selectivity is inactive."

DEFVAL        { -1 }

::={pktClassEntry 12}

pktClassDstPortHigh OBJECT-TYPE

SYNTAX              Integer32 (-1..65535)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object specifies the high range of the destination

port to which a packet is compared. A value of -1 indicates

that the selectivity is inactive."

DEFVAL        { -1 }

::={pktClassEntry 13}

pktClassVlanUserPri OBJECT-TYPE

SYNTAX              Integer32 (-1..7)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object specifies the VLAN User Priority to which a

packet is compared. A value of -1 indicates that the

selectivity is  inactive."

DEFVAL        { -1 }

::={pktClassEntry 14}

pktClassPhbAssociation OBJECT-TYPE

SYNTAX              Integer32 (0..63)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Associate the filter entry to a specific PHB (refer to

phbIdentifier)."

::={pktClassEntry 15}

pktClassRowStatus OBJECT-TYPE

SYNTAX              RowStatus

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Standard SNMP row status"

::={pktClassEntry 16}

--===================================================================

-----PhbMappingTable

--===================================================================

phbMappingTable OBJECT-TYPE

SYNTAX              SEQUENCE OF PhbMappingEntry

MAX-ACCESS          not-accessible

STATUS              current

DESCRIPTION

"This table is a list of Per-Hop Behaviour (PHB) MIB entries.

It describes the PHB mapping to the Request Class."

::={rcstQos 2}

phbMappingEntry OBJECT-TYPE

SYNTAX            PhbMappingEntry

MAX-ACCESS        not-accessible

STATUS            current

DESCRIPTION

"An entry in the PHB mapping table."

INDEX   { phbIdentifier }

::= { phbMappingTable 1 }

PhbMappingEntry ::= SEQUENCE {

phbIdentifier               Integer32,

phbName                     DisplayString,

phbRequestClassAssociation  Integer32,

phbMappingRowStatus         RowStatus

}

phbIdentifier OBJECT-TYPE

SYNTAX              Integer32 (1..64)

MAX-ACCESS          not-accessible

STATUS              current

DESCRIPTION

"Identification of the Per-Hop Behaviour (PHB) (1..64)."

::={phbMappingEntry 1}

phbName OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"The name of the Per-Hop Behaviour (PHB)."

::={phbMappingEntry 2}

phbRequestClassAssociation OBJECT-TYPE

SYNTAX              Integer32 (1..16)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object is an association of this Per-Hop Behaviour

(PHB) to a Request class (by reference to a Request Class

index)."

::={phbMappingEntry 3}

phbMappingRowStatus OBJECT-TYPE

SYNTAX              RowStatus

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Standard SNMP row status"

DEFVAL { active }

::={phbMappingEntry 4}

--===================================================================

--   RequestClassTable

--===================================================================

requestClassTable OBJECT-TYPE

SYNTAX              SEQUENCE OF RequestClassEntry

MAX-ACCESS          not-accessible

STATUS              current

DESCRIPTION

"This table is a list of Request class entries. This class

describes the layer 2 QoS objects."

::={rcstQos 3}

requestClassEntry OBJECT-TYPE

SYNTAX            RequestClassEntry

MAX-ACCESS        not-accessible

STATUS            current

DESCRIPTION

"An entry in the Request Class table."

INDEX   { requestClassIndex }

::= { requestClassTable 1 }

RequestClassEntry ::= SEQUENCE {

requestClassIndex               Integer32,

requestClassName                DisplayString,

requestClassChanId              Integer32,

requestClassVccVpi              Integer32,

requestClassVccVci              Integer32,

requestClassPidPoolReference    Integer32,

requestClassCra                 Integer32,

requestClassRbdcMax             Integer32,

requestClassRbdcTimeout         Integer32,

requestClassVbdcMax             Integer32,

requestClassVbdcTimeout         Integer32,

requestClassVbdcMaxBackLog      Integer32,

requestClassRowStatus           RowStatus

}

requestClassIndex OBJECT-TYPE

SYNTAX              Integer32 (1..16)

MAX-ACCESS          not-accessible

STATUS              current

DESCRIPTION

"Index of the Request Class table. A total of 16 entries are

supported."

::={requestClassEntry 1}

requestClassName OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Name of the Request Class."

::={requestClassEntry 2}

requestClassChanId OBJECT-TYPE

SYNTAX              Integer32 (0..15)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Channel id of the Request Class."

::={requestClassEntry 3}

requestClassVccVpi OBJECT-TYPE

SYNTAX              Integer32 (0..255)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Defines VPI used for the Request Class (ATM profile)."

::={requestClassEntry 4}

requestClassVccVci OBJECT-TYPE

SYNTAX              Integer32 (0..65535)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Defines VCI used for the Request Class (ATM profile)."

::={requestClassEntry 5}

requestClassPidPoolReference OBJECT-TYPE

SYNTAX              Integer32 (1..16)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Reference to the Packet IDentifier (PID) pool applicable

for the Request Class."

::={requestClassEntry 6}

requestClassCra OBJECT-TYPE

SYNTAX              Integer32

UNITS               "bits/s"

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Define Continuous Rate Assignment (CRA) level for the

Request Class in bit per second (bits/s)."

::={requestClassEntry 7}

requestClassRbdcMax OBJECT-TYPE

SYNTAX              Integer32

UNITS               "x2 kbits/s"

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Maximum Rate-Based Dynamic Capacity (RBDC) that can be

requested for the Request Class, in number of 2 kbits/s"

::={requestClassEntry 8}

requestClassRbdcTimeout OBJECT-TYPE

SYNTAX              Integer32

UNITS               "superframes"

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Persistence of the Rate-Based Dynamic Capacity (RBDC)

request, expressed in superframes"

::={requestClassEntry 9}

requestClassVbdcMax OBJECT-TYPE

SYNTAX              Integer32

UNITS               "ATM cells/MPEG packets"

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Maximum Volume-Based Dynamic Capacity (VBDC) that can be

allocated to the Request Class, in payload units (one ATM

cell or one MPEG packet) per superframe"

::={requestClassEntry 10}

requestClassVbdcTimeout OBJECT-TYPE

SYNTAX              Integer32

UNITS               "superframes"

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Time after which the RCST considers that the pending

requests are lost. The RCST may issue new requests for that

traffic. Volume-Based Dynamic Capacity (VBDC) Timeout is

expressed in superframes."                      ::={requestClassEntry 11}

requestClassVbdcMaxBackLog OBJECT-TYPE

SYNTAX              Integer32

UNITS               "bytes"

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

“Volume-Based Dynamic Capacity (VBDC) back log per Request

Class (expressed in bytes)"

::={requestClassEntry 12}

requestClassRowStatus OBJECT-TYPE

SYNTAX              RowStatus

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Standard SNMP row status"

DEFVAL { active }

::={requestClassEntry 13}

--===================================================================

-- The table of PID pools

--===================================================================

pidPoolTable OBJECT-TYPE

SYNTAX              SEQUENCE OF PidPoolEntry

MAX-ACCESS          not-accessible

STATUS              current

DESCRIPTION

"This table contains the Packet IDentifier (PID) pools.

For MPEG profile several Request Classes may be mapped

within a pool of several PIDs to allow Section Packing

across Several Request Classes.

A PID value may occur in more than one PID pool. Each PID

value can effectively occur only once in each pool."

::={rcstQos 4}

pidPoolEntry OBJECT-TYPE

SYNTAX            PidPoolEntry

MAX-ACCESS        not-accessible

STATUS            current

DESCRIPTION

"An entry in the PID pool table."

INDEX   { pidPoolIndex, pidIndex }

::= { pidPoolTable 1 }

PidPoolEntry ::= SEQUENCE {

pidPoolIndex            Integer32,

pidIndex                Integer32,

pidValue                Integer32,

pidPoolRowStatus        RowStatus

}

pidPoolIndex OBJECT-TYPE

SYNTAX              Integer32 (1..16)

MAX-ACCESS          not-accessible

STATUS              current

DESCRIPTION

"Index of the PID pool in the PID pool table."

::={pidPoolEntry 1}

pidIndex OBJECT-TYPE

SYNTAX              Integer32 (1..16)

MAX-ACCESS          not-accessible

STATUS              current

DESCRIPTION

"Index of the PID entry within the PID pool."

::={pidPoolEntry 2}

pidValue OBJECT-TYPE

SYNTAX              Integer32 (0..8191)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Defines one of the PIDs to be used in a PID pool of

pidPoolIndex.

A PID value may occur in more than one PID pool. Each PID

value can effectively occur only once in each pool."

::={pidPoolEntry 3}

pidPoolRowStatus OBJECT-TYPE

SYNTAX              RowStatus

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Standard SNMP row status"

DEFVAL { active }

::={pidPoolEntry 4}

qosGlobalRbdcMax OBJECT-TYPE

SYNTAX              Integer32

UNITS               "x2 kbits/s"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Global maximum RBDC that can be requested for the RCST, in

number of 2 kbits/s."

::={rcstQos 5}

qosGlobalVbdcMax OBJECT-TYPE

SYNTAX              Integer32

UNITS               "ATM cells/MPEG packets"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Global maximum VBDC that can be allocated to the RCST, in

payload units (one ATM cell or one MPEG packet) per

superframe."

::={rcstQos 6}

qosGlobalVbdcMaxBackLog OBJECT-TYPE

SYNTAX              Integer32

UNITS               "bytes"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Global VBDC back log at RCST level (expressed in bytes). It

is used only if the VBDC back log is not configured in the

Request class (expressed in bytes)."

::={rcstQos 7}

qosChannelIdStrictDispatching OBJECT-TYPE

SYNTAX              INTEGER {

notStrict (0),

strict (1)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Indicates whether the RCST will strictly follow RC

association when signaled through Channel_ID in the TBTP:

(0)- no strict association

(1)- strict association"

::={rcstQos 8}

--===================================================================

-- rcstControl sub-tree object types

--===================================================================

ctrlRebootCommand OBJECT-TYPE

SYNTAX           INTEGER {

idle        (1),

normal      (2),

alternate   (3)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This variable shall force the RCST to reboot

(1)- idle

(2)- normal reboot (from current software load)

(3)- reboot from alternate load (swap to alternate load

before reboot)"

DEFVAL {1}

::={rcstControl 1}

ctrlRcstTxDisable OBJECT-TYPE

SYNTAX           INTEGER {

idle        (1),

disable     (2)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This variable shall force the RCST to stop transmission

(transmit disabled as defined in SatLabs System

Recommendations):

(1)- idle

(2)- initiate Tx Disabled"

DEFVAL {1}

::={rcstControl 2}

ctrlUserTrafficDisable OBJECT-TYPE

SYNTAX           INTEGER {

idle        (1),

disable     (2)

}

MAX-ACCESS       read-write

STATUS           current

DESCRIPTION

"This variable shall disable user traffic (only RCST

management traffic can be transmitted)

(1)- idle

(2)- disable user traffic"

DEFVAL {1}

::={rcstControl 3}

ctrlCwEnable OBJECT-TYPE

SYNTAX           INTEGER {

off    (1),

on     (2)

}

MAX-ACCESS       read-write

STATUS           current

DESCRIPTION

"This variable will force the RCST to start transmission of

CW, if the RCST is first set to the installation state, and

is properly configured for CW transmission:

(1)- off

(2)- on"

DEFVAL {1}

::={rcstControl 4}

ctrlOduTxReferenceEnable OBJECT-TYPE

SYNTAX           INTEGER {

off    (1),

on     (2)

}

MAX-ACCESS       read-write

STATUS           current

DESCRIPTION

"Enables activation and deactivation of 10 MHz reference

clock in the Tx IFL cable:

(1) off

(2) on"

DEFVAL {2}

::={rcstControl 5}

ctrlOduTxDCEnable OBJECT-TYPE

SYNTAX           INTEGER {

off    (1),

on     (2)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Enables activation and deactivation of DC in the Tx IFL

cable:

(1) off

(2) on"

DEFVAL {2}

::={rcstControl 6}

ctrlOduRxDCEnable OBJECT-TYPE

SYNTAX           INTEGER {

off    (1),

on     (2)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Enables activation and deactivation of DC in the Rx IFL

cable:

(1) off

(2) on"

DEFVAL {2}

::={rcstControl 7}

ctrlDownloadFileCommand OBJECT-TYPE

SYNTAX           INTEGER {

idle (1),

config (2),

installationLog (3)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This variable will initiate a RCST configuration file

download process

(1) idle

(2) download RCST configuration file from TFTP/FTP server

(3) download RCST installation log file from TFTP/FTP server

(INSTALL_LOG Option)"

DEFVAL {1}

::={rcstControl 8}

ctrlUploadFileCommand OBJECT-TYPE

SYNTAX           INTEGER {

idle              (1),

config            (2),

eventAlarm        (3),

installationLog   (4)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This variable will initiate a RCST upload process

(1) idle

(2) upload RCST configuration file to TFTP/FTP server

(3) upload RCST event/alarm log file to TFTP/FTP server

(4) upload RCST installation log file to TFTP/FTP server

(INSTALL_LOG Option)"

DEFVAL {1}

::={rcstControl 9}

ctrlActivateConfigFileCommand OBJECT-TYPE

SYNTAX           INTEGER {

idle            (1),

activate        (2)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Triggers the RCST to use the configuration file and update

its parameters accordingly.  Some RCST implementations may

require a reboot for the parameters to take effect (vendor

specific).

(1)    Idle

(2)    activate"

DEFVAL {1}

::={rcstControl 10}

ctrlRcstLogonCommand OBJECT-TYPE

SYNTAX           INTEGER {

idle           (1),

logon          (2)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This variable will initiate a RCST logon

(1) idle

(2) initiate RCST logon"

DEFVAL {1}

::={rcstControl 11}

ctrlRcstLogoffCommand OBJECT-TYPE

SYNTAX           INTEGER {

idle      (1),

logoff    (2)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This variable will initiate a RCST logoff

(1) idle

(2) initiate RCST logoff"

DEFVAL {1}

::={rcstControl 12}

--===================================================================

-- rcstState sub-tree object types

--===================================================================

rcstMode OBJECT-TYPE

SYNTAX              INTEGER {

installation (0),

operational (1)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Identifies the current mode of the RCST is and allows it to

return to the installation mode when needed. Values for the

RCST mode are:

Installation (0)

Operational (1)"

::={rcstState 1}

rcstFaultStatus OBJECT-TYPE

SYNTAX              INTEGER {

nofault (0),

fault (1)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Provide the fault status of the terminal. The fault status

management is vendor specific. Values for the Fault Status

are:

no fault (0)

fault (1)"

::={rcstState 2}

rcstFwdLinkStatus OBJECT-TYPE

SYNTAX              INTEGER {

notAcquired (0),

acquired     (1)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Provides the status of the RCST Forward Link. Values for the

Forward Link Status are:

Not acquired (0)

Acquired (1)"

::={rcstState 3}

rcstRtnLinkStatus OBJECT-TYPE

SYNTAX              INTEGER {

loggedOff (0),

loggedOn  (1)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Provides the status of the RCST Return Link. Values for the

Return Link Status are:

Logged-off (0)

Logged-on (1)"

::={rcstState 4}

rcstLogUpdated OBJECT-TYPE

SYNTAX              INTEGER {

noUpdate         (0),

logfileUpdated  (1)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates the existence of an updated event log file:

No update (0)

Event Log file updated (1)

The RCST should remove the Event Log file updated indication

as the log file is fetched by the NCC."

::={rcstState 5}

rcstCurrentSoftwareVersion OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Current RCST software version."

::={rcstState 6}

rcstAlternateSoftwareVersion OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Alternate (backup/new) RCST software version."

::={rcstState 7}

rcstActivatedConfigFileVersion OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Version of the most recently activated configuration file.

The version is vendor specific."

::={rcstState 8}

rcstDownloadedConfigFileVersion OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Version of the most recently downloaded configuration file.

Version is vendor specific. If the value is different from

rcstActivatedConfigFileVersion, it is pending for activation."

::={rcstState 9}

--===================================================================

--- fwdConfig sub-tree object types

--=====================================================================

fwdStartTable OBJECT-TYPE

SYNTAX              SEQUENCE OF FwdStartEntry

MAX-ACCESS          not-accessible

STATUS              current

DESCRIPTION

"Lists Forward Links attachment points (e.g. different for

installation and operation).

The table describes the forward link parameters used for the

start-up stream with the NCC."

::={fwdConfig 1}

fwdStartEntry OBJECT-TYPE

SYNTAX            FwdStartEntry

MAX-ACCESS        not-accessible

STATUS            current

DESCRIPTION

"An entry in the Forward Link StartConfig table."

INDEX   { fwdStartIndex }

::= { fwdStartTable 1 }

FwdStartEntry ::= SEQUENCE {

fwdStartIndex        Integer32,

fwdStartPopId        Integer32,

fwdStartFrequency     Integer32,

fwdStartPolar         INTEGER,

fwdStartFormat        INTEGER,

fwdStartRolloff       INTEGER,

fwdStartSymbolRate    Integer32,

fwdStartInnerFec      INTEGER,

fwdStartRowStatus     RowStatus

}

fwdStartIndex OBJECT-TYPE

SYNTAX              Integer32 (1..8)

MAX-ACCESS          not-accessible

STATUS              current

DESCRIPTION

"Index of the Forward Link StartConfig table."

::={fwdStartEntry 1}

fwdStartPopId OBJECT-TYPE

SYNTAX              Integer32 (-1..65535)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Population identifier associated with the start-up

forward link:

-1: any (auto)

0-65535: specific StartPopId

If 'any' is set, the RCST will assume membership of any

announced population ID and will commence with logon in

accordance with this assumption."

::={fwdStartEntry 2}

fwdStartFrequency OBJECT-TYPE

SYNTAX              Integer32

UNITS               "x100 kHz"

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Frequency of the start transponder carrying a

Network Information Table to which any RCST shall

trigger to acquire forward link. Its value shall be given

in multiple of 100 kHz."

::={fwdStartEntry 3}

fwdStartPolar OBJECT-TYPE

SYNTAX              INTEGER   {

linearHorizontal (0),

linearVertical   (1),

circularLeft     (2),

circularRight    (3)

}

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"2-bit field giving the polarization of the start

transponder carrying an Network Information Table

to which any RCST shall trigger to acquire forward link:

00: linear and horizontal

01: linear and vertical

10: circular left

11: circular right"

::={fwdStartEntry 4}

fwdStartFormat OBJECT-TYPE

SYNTAX        INTEGER {

auto      (-1),

dvbs      (0),

dvbs2ccm  (1),

dvbs2acm  (2)

}

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Specifies the transmission format standard applied for the

startup stream. The start transport stream carries a Network

Information Table that the RCST uses for acquiring the

forward link signaling. Supported values are:

-1: unspecified (automatic format acquisition is

assumed)

0: DVB-S (support of this value is mandatory if DVB-S

support is claimed)

1: DVB-S2 with CCM (support of this value is mandatory

if DVB-S2 CCM support is claimed)

2: DVB-S2 with VCM or ACM (support of this value is

mandatory if DVB-S2 ACM support is claimed)

This allows the RCST to discriminate between CCM and VCM/ACM

when selecting FL.

The support of automatic format selection is optional. One

Or several of the other format selections must be supported,

according to the claimed SatLabs profile support."

::={fwdStartEntry 5}

fwdStartRolloff OBJECT-TYPE

SYNTAX        INTEGER {

autoRolloff     (0),

rolloff020     (1),

rolloff025     (2),

rolloff035    (3)

}

MAX-ACCESS    read-create

STATUS        current

DESCRIPTION

"Specifies the receive filter roll-off applied on the start

transponder. The start transponder carries a Network

Information Table that the RCST uses for acquiring the

forward link signaling.

Supported values are:

0: any (auto)

1: 0.20

2: 0.25

3: 0.35"

::={fwdStartEntry 6}

fwdStartSymbolRate OBJECT-TYPE

SYNTAX              Integer32

UNITS               "x100 symbols/s"

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Specifies the symbol rate on the start transponder

carrying a Network Information Table to which any RCST

shall trigger to acquire forward link. Its value shall be

given in multiple of 100 symbols/s."

::={fwdStartEntry 7}

fwdStartInnerFec OBJECT-TYPE

SYNTAX              INTEGER     {

autoFec       (-1),

fecRate12    (0),

fecRate23    (1),

fecRate34    (2),

fecRate56    (3),

fecRate78    (4),

fecRate89    (5),

fecRate35    (6),

fecRate45    (7),

fecRate910   (8),

fecRate25    (9),

fecRate13    (10),

fecRate14    (11),

noInnerCode   (12)                }

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Specifies the inner Forward Error Correction used on the

start transponder carrying a Network Information Table

to which any RCST shall trigger to acquire forward link.

Supported values are:

autoFec      (-1),

fecRate1/2    (0),

fecRate2/3    (1),

fecRate3/4    (2),

fecRate5/6    (3),

fecRate7/8    (4),

fecRate8/9    (5),

fecRate3/5    (6),

fecRate4/5    (7),

fecRate9/10   (8),

fecRate2/5    (9),

fecRate1/3    (10),

fecRate1/4    (11),

noInnerCode   (12)

The support of autoFec is optional."

::={fwdStartEntry 8}

fwdStartRowStatus OBJECT-TYPE

SYNTAX              RowStatus

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Standard SNMP row status"

DEFVAL  { active }

::={fwdStartEntry 9}

--===================================================================

--  fwdStatus sub-tree object types

--===================================================================

fwdStatusPopId OBJECT-TYPE

SYNTAX              Integer32 (0..65535)

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Population identifier applied at log-on:

0-65535: specific StartPopId

If the RCST was allowed to logon with any population, the

RCST will report the base number of the announced

population ID indicated by the RCS Map Table linkage

descriptor used at logon."

::={fwdStatus 1}

fwdStatusTable OBJECT-TYPE

SYNTAX              SEQUENCE OF FwdStatusEntry

MAX-ACCESS          not-accessible

STATUS              current

DESCRIPTION

"This table describes the current status of Forward Link

interfaces."

::={fwdStatus 2}

fwdStatusEntry OBJECT-TYPE

SYNTAX            FwdStatusEntry

MAX-ACCESS        not-accessible

STATUS            current

DESCRIPTION

"An entry in the Forward Link Status table. Each entry is

associated with a physical interface.

A RCST shall support at least one entry."

INDEX   { fwdStatusIndex }

::= { fwdStatusTable 1 }

FwdStatusEntry ::= SEQUENCE {

fwdStatusIndex             Integer32,

fwdStatusIfReference       Integer32,

fwdStatusNetId             Integer32,

fwdStatusNetName           DisplayString,

fwdStatusFormat            INTEGER,

fwdStatusFrequency         Integer32,

fwdStatusPolar             INTEGER,

fwdStatusInnerFec          INTEGER,

fwdStatusSymbolRate        Integer32,

fwdStatusRolloff           INTEGER,

fwdStatusModulation        INTEGER,

fwdStatusFecFrame          INTEGER,

fwdStatusPilot             INTEGER,

fwdStatusBer               Integer32,

fwdStatusCnr               Integer32,

fwdStatusRxPower           Integer32

}

fwdStatusIndex OBJECT-TYPE

SYNTAX              Integer32 (1..8)

MAX-ACCESS          not-accessible

STATUS              current

DESCRIPTION

"Index of the Forward Link Status table."

::={fwdStatusEntry 1}

fwdStatusIfReference OBJECT-TYPE

SYNTAX              Integer32 (1..8)

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Cross reference to the interface table"

::={fwdStatusEntry 2}

fwdStatusNetId  OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Interactive network identifier of the forward

link (from RCS Map Table)"

::={fwdStatusEntry 3}

fwdStatusNetName OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"The name of the interactive network of the forward

link (from RCS Map Table)"

::={fwdStatusEntry 4}

fwdStatusFormat OBJECT-TYPE

SYNTAX           INTEGER  {

dvbs         (0),

dvbs2ccm     (1),

dvbs2acm     (2),

reservedFormat  (3)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Specifies the transmission format applied on the forward

link. Supported values are (from RMT):

0: DVB-S

1: DVB-S2 using CCM

2: DVB-S2 using VCM or ACM

3: reserved"

::={fwdStatusEntry 5}

fwdStatusFrequency OBJECT-TYPE

SYNTAX              Integer32

UNITS               "x100 kHz"

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"An estimate of the frequency of the forward link. Its value

shall be given in multiple of 100 kHz."

::={fwdStatusEntry 6}

fwdStatusPolar OBJECT-TYPE

SYNTAX              INTEGER   {

linearHorizontal (0),

linearVertical   (1),

circularLeft     (2),

circularRight    (3)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"2-bit field giving the polarization of the forward link

(from RMT):

00: linear and horizontal

01: linear and vertical

10: circular left

11: circular right"

::={fwdStatusEntry 7}

fwdStatusInnerFec OBJECT-TYPE

SYNTAX              INTEGER     {

unknown      (-1),

fecRate12    (0),

fecRate23    (1),

fecRate34    (2),

fecRate56    (3),

fecRate78    (4),

fecRate89    (5),

fecRate35    (6),

fecRate45    (7),

fecRate910   (8),

fecRate25    (9),

fecRate13    (10),

fecRate14    (11),

noInnerCode   (12)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Specifies the inner Forward Error Correction used on the

forward link for transmission to the RCST.

Supported values are:

unknown      (-1),

fecRate1/2    (0),

fecRate2/3    (1),

fecRate3/4    (2),

fecRate5/6    (3),

fecRate7/8    (4),

fecRate8/9    (5),

fecRate3/5    (6),

fecRate4/5    (7),

fecRate9/10   (8),

fecRate2/5    (9),

fecRate1/3    (10),

fecRate1/4    (11),

noInnerCode   (12)

The RCST will report a value that has been used for transmission

to the RCST within the most recent 60 seconds.

If this is not relevant, the RCST will report 'unknown'."

::={fwdStatusEntry 8}

fwdStatusSymbolRate OBJECT-TYPE

SYNTAX              Integer32

UNITS               "x100 symbols/s"

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"An estimate of the symbol rate of the forward link.

Its value shall be given in multiple of 100 symbol/s."

::={fwdStatusEntry 9}

fwdStatusRolloff OBJECT-TYPE

SYNTAX              INTEGER   {

undefRolloff       (0),

rolloff020         (1),

rolloff025         (2),

rolloff035         (3)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"An estimate of the roll-off applied on the forward link.

Supported values are:

0: undefined

1: 0.20

2: 0.25

3: 0.35"

::={fwdStatusEntry 10}

fwdStatusModulation OBJECT-TYPE

SYNTAX              INTEGER   {

unknown        (0),

mBPSK          (1),

mQPSK          (2),

m8PSK          (3),

m16APSK        (4),

m32APSK        (5)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates the modulation on the forward link used for

transmission to the RCST.

0: unknown

1: BPSK

2: QPSK

3: 8PSK

4: 16APSK

5: 32APSK

The RCST will report a value that has been used for

transmission to the RCST within the most recent 60 seconds.

If this is not relevant, the RCST will report 'unknown'."

::={fwdStatusEntry 11}

fwdStatusFecFrame OBJECT-TYPE

SYNTAX              INTEGER {

unknown       (0),

shortframe    (1),

longframe     (2)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates the frame length used on the forward link for

transmission to the RCST.

Supported values are:

0: Unknown

1: Short frame

2: Normal frame

The RCST will report a value that has been used for

transmission to the RCST within the most recent 60 seconds.

If this is not relevant, the RCST will report 'unknown'."

::={fwdStatusEntry 12}

fwdStatusPilot OBJECT-TYPE

SYNTAX              INTEGER {

unknown      (0),

pilotNotused (1),

pilotUsed    (2)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates whether pilots are used on the forward link for

transmission to the RCST.

Supported values are:

0: Unknown

1: Pilots are not used

2: Pilots are used

The RCST will report a value that has been used for

transmission to the RCST within the most recent 60 seconds.

If this is not relevant, the RCST will report 'unknown'."

::={fwdStatusEntry 13}

fwdStatusBer OBJECT-TYPE

SYNTAX              Integer32

UNITS               "exponent of 10"

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Provides the RCST BER on the Forward Link in log10 units."

::={fwdStatusEntry 14}

fwdStatusCnr OBJECT-TYPE

SYNTAX              Integer32

UNITS               "0.1 dB"

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Provides the RCST CNR on the Forward Link in 0.1 dB units."

::={fwdStatusEntry 15}

fwdStatusRxPower OBJECT-TYPE

SYNTAX              Integer32

UNITS               "0.1 dBm"

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Provides the power level of the Forward Link as received at

the IDU, in 0.1 dBm units."

DEFVAL { -500 }

::={fwdStatusEntry 16}

--===================================================================

--  rtnConfig sub-tree object types

--===================================================================

rtnConfigMaxEirp OBJECT-TYPE

SYNTAX              Integer32

UNITS               "x0.1 dBm"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Max EIRP of the RCST given in resolution of 0.1 dBm,

applied when the IDU can itself set the necessary IDU TX

output level e.g. when using a BUC that has a power level

detector and provides sufficient feedback to the IDU."

::= {rtnConfig 1}

rtnConfigDefIfLevel OBJECT-TYPE

SYNTAX              Integer32

UNITS               "x0.1 dBm"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"IDU TX output level applied in case the rtnConfigMaxEirp

cannot be used. The resolution is 0.1 dBm and the accuracy

is +/- 1 dBm."

::= {rtnConfig 2}

--===================================================================

--  rtnStatus sub-tree object types

--===================================================================

rtnStatusEbN0 OBJECT-TYPE

SYNTAX              Integer32

UNITS               "x0.1 dB"

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"The EbN0 value reported for the return link, referenced to

the regular SYNC burst transmission, in 0.1 dB units."

::= {rtnStatus 1}

rtnStatusSFDuration OBJECT-TYPE

SYNTAX              Integer32 (250..7500)

UNITS               "0.1 ms"

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"The duration of the currently applied return link super-

frame structure, in tenths of milliseconds."

::= {rtnStatus 2}

rtnStatusPayloadUnit OBJECT-TYPE

SYNTAX              INTEGER   {

unitATM    (0),

unitMPEG   (1)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates if the payload unit used for the return link is

ATM or MPEG."

::= {rtnStatus 3}

-==================================================================

--    conformance information

--==================================================================

rcstGroups            OBJECT IDENTIFIER ::= {dvbrcsConformance 1}

rcstCompliances       OBJECT IDENTIFIER ::= {dvbrcsConformance 2}

--==================================================================

--    conformance statements

--==================================================================

rcstCompliance1 MODULE-COMPLIANCE

STATUS          current

DESCRIPTION

"The compliance statement for DVB-RCS terminals that are

compliant with SatLabs System Recommendations. "

MODULE  -- this module

MANDATORY-GROUPS    {rcstSystemGroup, rcstNetworkGroup, rcstInstallGroup, rcstQosGroup, rcstControlGroup, rcstStateGroup, fwdConfigGroup, fwdStatusGroup, rtnConfigGroup, rtnStatusGroup}

GROUP       rcstExtSystemGroup

DESCRIPTION "This group is mandatory for an RCST

that claims to support the SNMPMISC option."

GROUP       rcstExtNetworkGroup

DESCRIPTION "This group is mandatory for an RCST

that claims to support the SNMPMISC option."

GROUP       rcstDnsGroup

DESCRIPTION "This group is mandatory for an RCST

that claims to support the DNS option."

GROUP       rcstExtInstallGroup

DESCRIPTION "This group is mandatory for an RCST

that claims to support the INSTALL_LOG feature."

GROUP       rcstEnhancedClassifierGroup

DESCRIPTION "This group is mandatory for an RCST

that claims to support the ENHCLASSIFIER feature."

GROUP       rcstMpegQosGroup

DESCRIPTION "This group is mandatory for an RCST

that claims to support the MPEG_TRF option."

GROUP       rcstGlobalQosGroup

DESCRIPTION "This group is mandatory for an RCST

that claims to support the RCST_PARA feature."

GROUP       rcstStrictQosGroup

DESCRIPTION "This group is mandatory for an RCST

that claims to support the CHID_STRICT option."

GROUP       rcstExtControlGroup

DESCRIPTION "This group is mandatory for an RCST

that claims to support the SNMPMISC option."

GROUP       rtnExtConfigGroup

DESCRIPTION "This group is mandatory for an RCST

that claims to support the SNMPMISC option."

GROUP       rtnExtStatusGroup

DESCRIPTION "This group is mandatory for an RCST

that claims to support the SNMPMISC option."

OBJECT systemOduAntennaGain

MIN-ACCESS read-only

DESCRIPTION    "Write access only required if SNMPMISC

support is claimed."

OBJECT systemOduTxType

MIN-ACCESS read-only

DESCRIPTION    "Write access required if the RCST does

not provide the information."

OBJECT systemOduRxType

MIN-ACCESS read-only

DESCRIPTION    "Write access required if the RCST does

not provide the information."

OBJECT systemOduRxBand

MIN-ACCESS read-only

DESCRIPTION    "Write access only required if SNMPMISC

support is claimed."

OBJECT systemOduRxLO

MIN-ACCESS read-only

DESCRIPTION    "Write access only required if SNMPMISC

support is claimed."

OBJECT systemOduTxLO

MIN-ACCESS read-only

DESCRIPTION    "Write access only required if SNMPMISC

support is claimed."

OBJECT pktClassDscpLow

MIN-ACCESS read-only

DESCRIPTION    "Create access only required if

ENHCLASSIFIER support is claimed."

OBJECT pktClassDscpHigh

MIN-ACCESS read-only

DESCRIPTION    "Create access only required if

ENHCLASSIFIER support is claimed."

OBJECT pktClassDscpMarkValue

MIN-ACCESS read-only

DESCRIPTION    "Create access only required if

ENHCLASSIFIER support is claimed."

OBJECT phbName

MIN-ACCESS read-only

DESCRIPTION    "Create access only required if SNMPMISC

support is claimed."

OBJECT phbRequestClassAssociation

MIN-ACCESS read-only

DESCRIPTION    "Create access only required if SNMPMISC

support is claimed."

OBJECT phbMappingRowStatus

MIN-ACCESS read-only

DESCRIPTION    "Create access only required if SNMPMISC

support is claimed."

OBJECT requestClassName

MIN-ACCESS read-only

DESCRIPTION    "Write access only required if SNMPMISC

support is claimed."

OBJECT requestClassChanId

MIN-ACCESS read-only

DESCRIPTION    "Write access only required if SNMPMISC

support is claimed."

OBJECT requestClassVccVpi

MIN-ACCESS read-only

DESCRIPTION    "Write access only required if SNMPMISC

support is claimed."

OBJECT requestClassVccVci

MIN-ACCESS read-only

DESCRIPTION    "Write access only required if SNMPMISC

support is claimed."

OBJECT requestClassPidPoolReference

MIN-ACCESS not-accessible

DESCRIPTION    "Read-only access required if MPEG option

is claimed. Write access only required if also SNMPMISC

support is claimed."

OBJECT requestClassCra

MIN-ACCESS read-only

DESCRIPTION    "Write access only required if SNMPMISC

support is claimed."

OBJECT requestClassRbdcMax

MIN-ACCESS read-only

DESCRIPTION    "Write access only required if SNMPMISC

support is claimed."

OBJECT requestClassRbdcTimeout

MIN-ACCESS read-only

DESCRIPTION    "Write access only required if SNMPMISC

support is claimed."

OBJECT requestClassVbdcMax

MIN-ACCESS read-only

DESCRIPTION    "Write access only required if SNMPMISC

support is claimed."

OBJECT requestClassVbdcTimeout

MIN-ACCESS read-only

DESCRIPTION    "Write access only required if SNMPMISC

support is claimed."

OBJECT requestClassVbdcMaxBackLog

MIN-ACCESS read-only

DESCRIPTION    "Write access only required if SNMPMISC

support is claimed."

OBJECT requestClassRowStatus

MIN-ACCESS read-only

DESCRIPTION    "Write access only required if SNMPMISC

support is claimed."

OBJECT pidValue

MIN-ACCESS not-accessible

DESCRIPTION    "Read-only access required if MPEG option

is claimed.  Write access only required if also SNMPMISC

support is claimed."

OBJECT pidPoolRowStatus

MIN-ACCESS not-accessible

DESCRIPTION    "Read-only access required if MPEG option

is claimed.  Write access only required if also SNMPMISC

support is claimed."

::= { rcstCompliances 1 }

===================================================================

--    units of conformance

--

====================================================================

===================================================================

--    object groups for rcst system

--===================================================================

rcstSystemGroup OBJECT-GROUP

OBJECTS {

systemMibRevision,

systemSatLabsProfilesDeclaration,

systemSatLabsOptionsDeclaration,

systemSatLabsFeaturesDeclaration,

systemLocation,

systemOduAntennaGain,

systemOduTxType,

systemOduRxType,

systemOduRxBand,

systemOduRxLO,

systemOduTxLO,

tcpPep,

httpPep

}

STATUS    current

DESCRIPTION    "A collection of objects providing information

applicable for basic device management support."

::= { rcstGroups 1 }

rcstExtSystemGroup OBJECT-GROUP

OBJECTS {

systemOduAntennaSize,

systemOduSspa

}

STATUS    current

DESCRIPTION    "A collection of objects providing information

applicable for extended device management support."

::= { rcstGroups 2 }

====================================================================

--    object groups for rcst networking

--====================================================================

rcstNetworkGroup OBJECT-GROUP

OBJECTS {

networkOamIpAddress,

networkOamIpNetworkMask,

networkLanIpAddress,

networkLanIpNetworkMask,

networkConfigFileDownloadUrl,

networkConfigFileUploadUrl,

networkLogFileUploadUrl

}

STATUS    current

DESCRIPTION    "A collection of objects providing basic networking management support."

::= { rcstGroups 3 }

rcstExtNetworkGroup OBJECT-GROUP

OBJECTS {

networkOamIpAddressAssign,

networkAirInterfaceDefaultGateway,

networkNccMgtIpAddress

}

STATUS    current

DESCRIPTION    "A collection of objects providing extended

networking management support."

::= { rcstGroups 4 }

rcstDnsGroup OBJECT-GROUP

OBJECTS {

primaryDnsServerIpAddress,

secondaryDnsServerIpAddress

}

STATUS    current

DESCRIPTION    "IP addresses of the primary and secondary DNS

servers to be used to resolve URL's as needed."

::= { rcstGroups 5 }

====================================================================

--    object groups for rcst installation

--====================================================================

rcstInstallGroup OBJECT-GROUP

OBJECTS {

installAntennaAlignmentState,

installCwFrequency,

installCwMaxDuration,

installCwPower,

installCoPolReading,

installXPolReading,

installCoPolTarget,

installXPolTarget,

installStandByDuration,

installTargetEsN0

}

STATUS    current

DESCRIPTION    "A collection of objects providing information

applicable for basic installation support."

::= { rcstGroups 6 }

rcstExtInstallGroup OBJECT-GROUP

OBJECTS {

networkInstallLogFileDownloadUrl,

networkInstallLogFileUploadUrl

}

STATUS    current

DESCRIPTION    "A collection of objects providing extended

device installation support."

::= { rcstGroups 7 }

==================================================================

--    object groups for QOS

--

==================================================================

rcstQosGroup OBJECT-GROUP

OBJECTS {

pktClassDscpLow,

pktClassDscpHigh,

pktClassDscpMarkValue,

pktClassPhbAssociation,

pktClassRowStatus,

phbName,

phbRequestClassAssociation,

phbMappingRowStatus,

requestClassName,

requestClassChanId,

requestClassVccVpi,

requestClassVccVci,

requestClassCra,

requestClassRbdcMax,

requestClassRbdcTimeout,

requestClassVbdcMax,

requestClassVbdcTimeout,

requestClassVbdcMaxBackLog,

requestClassRowStatus

}

STATUS    current

DESCRIPTION    "A collection of objects providing basic access to

QOS configuration data."

::= { rcstGroups 8 }

rcstEnhancedClassifierGroup OBJECT-GROUP

OBJECTS {

pktClassIpProtocol,

pktClassIpSrcAddress,

pktClassIpSrcAddressMask,

pktClassIpDstAddress,

pktClassIpDstAddressMask,

pktClassSrcPortLow,

pktClassSrcPortHigh,

pktClassDstPortLow,

pktClassDstPortHigh,

pktClassVlanUserPri

}

STATUS    current

DESCRIPTION    "A collection of objects providing support for

management of the enhanced classifier."

::= { rcstGroups 9 }

rcstMpegQosGroup OBJECT-GROUP

OBJECTS {

requestClassPidPoolReference,

pidValue,

pidPoolRowStatus

}

STATUS    current

DESCRIPTION    "A collection of objects providing access to MPEG

related link QOS configuration data."

::= { rcstGroups 10 }

rcstGlobalQosGroup OBJECT-GROUP

OBJECTS {

qosGlobalRbdcMax,

qosGlobalVbdcMax,

qosGlobalVbdcMaxBackLog

}

STATUS    current

DESCRIPTION    "A collection of objects providing access to

global RCST QOS configuration data."

::= { rcstGroups 11 }

rcstStrictQosGroup OBJECT-GROUP

OBJECTS {

qosChannelIdStrictDispatching

}

STATUS    current

DESCRIPTION    "A collection of objects allowing management of

strict channel ID dispatching."

::= { rcstGroups 12 }

==================================================================

--    object groups for rcst control

--==================================================================

rcstControlGroup OBJECT-GROUP

OBJECTS {

ctrlRebootCommand,

ctrlUserTrafficDisable,

ctrlCwEnable,

ctrlDownloadFileCommand,

ctrlUploadFileCommand,

ctrlActivateConfigFileCommand

}

STATUS    current

DESCRIPTION    "A collection of objects allowing basic RCST

control."

::= { rcstGroups 13 }

rcstExtControlGroup OBJECT-GROUP

OBJECTS {

ctrlRcstTxDisable,

ctrlOduTxReferenceEnable,

ctrlOduTxDCEnable,

ctrlOduRxDCEnable,

ctrlRcstLogonCommand,

ctrlRcstLogoffCommand

}

STATUS    current

DESCRIPTION    "A collection of objects allowing extended RCST

control."

::= { rcstGroups 14 }

==================================================================

--    object groups for rcst state

--

==================================================================

rcstStateGroup OBJECT-GROUP

OBJECTS {

rcstMode,

rcstFaultStatus,

rcstFwdLinkStatus,

rcstLogUpdated,

rcstCurrentSoftwareVersion,

rcstAlternateSoftwareVersion,

rcstActivatedConfigFileVersion,

rcstDownloadedConfigFileVersion

}

STATUS    current

DESCRIPTION    "A collection of objects allowing access to RCST

state."

::= { rcstGroups 15 }

==================================================================

--    object groups for forward link

--

==================================================================

fwdConfigGroup OBJECT-GROUP

OBJECTS {

fwdStartPopId,

fwdStartFrequency,

fwdStartPolar,

fwdStartFormat,

fwdStartRolloff,

fwdStartSymbolRate,

fwdStartInnerFec,

fwdStartRowStatus

}

STATUS    current

DESCRIPTION    "A collection of objects providing basic start

forward link configuration support."

::= { rcstGroups 16 }

fwdStatusGroup OBJECT-GROUP

OBJECTS {

fwdStatusPopId,

fwdStatusIfReference,

fwdStatusNetId,

fwdStatusNetName,

fwdStatusFormat,

fwdStatusFrequency,

fwdStatusPolar,

fwdStatusInnerFec,

fwdStatusSymbolRate,

fwdStatusRolloff,

fwdStatusModulation,

fwdStatusFecFrame,

fwdStatusPilot,

fwdStatusBer,

fwdStatusCnr,

fwdStatusRxPower

}

STATUS    current

DESCRIPTION    "A collection of objects providing forward link status."

::= { rcstGroups 17 }

==================================================================

--    object groups for return link

--

==================================================================

rtnConfigGroup OBJECT-GROUP

OBJECTS {

rtnConfigDefIfLevel

}

STATUS    current

DESCRIPTION    "A collection of objects providing basic return

link configuration support."

::= { rcstGroups 18 }

rtnExtConfigGroup OBJECT-GROUP

OBJECTS {

rtnConfigMaxEirp

}

STATUS    current

DESCRIPTION    "A collection of objects providing extended

return link configuration support."

::= { rcstGroups 19 }

rtnStatusGroup OBJECT-GROUP

OBJECTS {

rtnStatusPayloadUnit

}

STATUS    current

DESCRIPTION    "A collection of objects allowing access to return

link status."

::= { rcstGroups 20 }

rtnExtStatusGroup OBJECT-GROUP

OBJECTS {

rcstRtnLinkStatus,

rtnStatusEbN0,

rtnStatusSFDuration

}

STATUS    current

DESCRIPTION    "A collection of objects allowing access to

extended return link status."

::= { rcstGroups 21 }

END

# Security Considerations

Some of the readable objects in this MIB module (i.e., objects with a MAX-ACCESS other than not-accessible) may be considered sensitive or vulnerable in some network environments.

SNMP versions prior to SNMPv3 did not include adequate security. Even if the network itself is secure (for example by using IPsec), even then, there is no control as to who on the secure network is allowed to access and GET/SET (read/change/create/delete) the objects in this MIB module.

It is RECOMMENDED that implementers consider the security features provided by the SNMPv3 framework (see [RFC3410], section 8), including full support for the SNMPv3 cryptographic mechanisms (for authentication and privacy).

Further, deployment of SNMP versions prior to SNMPv3 is NOT RECOMMENDED.  Instead, it is RECOMMENDED to deploy SNMPv3 and to enable cryptographic security.  It is then a customer/operator responsibility to ensure that the SNMP entity giving access to an instance of this MIB module, is properly configured to give access to the objects only to those principals (users) that have legitimate rights to indeed GET or SET (change/create/delete) them.

# IANA Considerations

This document includes no request to IANA. The transmission and ifType numbers described in Section 3 have already been assigned under the smi-numbers registry.

# Acknowledgments

The authors thank Gorry Fairhurst for advice in the preparation of this document.

The authors recognize this document is a collective effort of the SatLabs Group (www.satlabs.org), in particular the many corrections and suggestions brought by Juan Luis Manas.

# References

## Normative References

[IANA]    Internet Assigned Numbers Authority, "Internet Assigned Numbers Authority", June 2008,               <http://www.iana.org/assignments/smi-numbers>.

[RFC2119] Bradner, S., "Key words for use in RFCs to Indicate Requirement Levels", BCP 14, RFC 2119, March 1997 (BEST CURRENT PRACTICE).

[RFC2578] McCloghrie, K., Ed., Perkins, D., Ed., and J. Schoenwaelder, Ed., "Structure of Management Information, Version 2 (SMIv2)", STD 58, RFC 2578, April 1999.

[RFC2579] McCloghrie, K., Ed., Perkins, D., Ed., and J.               Schoenwaelder, Ed., "Textual Conventions for SMIv2",               STD 58, RFC 2579, April 1999.

[RFC2580] McCloghrie, K., Perkins, D., and J. Schoenwaelder, "Conformance Statements for SMIv2", STD 58, RFC 2580, April 1999.

[RFC2863] 	K. McCloghrie, F. Kastenholz, "The Interfaces Group MIB", RFC 2863, June 2000.

## Informative References

[ISO-MPEG] ISO/IEC DIS 13818-1:2000, "Information Technology;                 Generic Coding of Moving Pictures and Associated Audio Information Systems", International Standardisation Organisation (ISO).

[ETSI-DAT] EN 301 192, "Digital Video Broadcasting (DVB); DVB Specifications for Data Broadcasting", European Telecommunications Standards Institute (ETSI).

[ETSI-DVBS] EN 301 421 "Digital Video Broadcasting (DVB); Modulation and Coding for DBS satellite systems at 11/12 GHz", European Telecommunications Standards Institute (ETSI).

[ETSI-DVBS2] ETSI EN 302 307 "Digital Video Broadcasting (DVB); Second generation framing structure, channel coding and modulation systems for Broadcasting, Interactive Services, News Gathering and other broadband satellite applications", European Telecommunications Standards Institute (ETSI).

[ETSI-GSE]		ETSI TS 102 606 "Digital Video Broadcasting (DVB); Generic Stream Encapsulation (GSE) Protocol", European Telecommunications Standards Institute (ETSI).

[ETSI-RCS] ETSI 301 791 "Digital Video Broadcasting (DVB); Interaction Channel for Satellite Distribution Systems", European Telecommunications Standards Institute (ETSI).

[ETSI-SI]	ETSI EN 300 468 "Digital Video Broadcasting (DVB); Specification for Service Information (SI) in DVB Systems", European Telecommunications Standards Institute (ETSI).

[RFC3410] Case, J., Mundy, R., Partain, D., and B. Stewart, "Introduction and Applicability Statements for Internet-Standard Management Framework", RFC 3410, December 2002.

[RFC4259] Montpetit, M.-J., Fairhurst, G., Clausen, H., Collini-Nocker, B., and H. Linder, "A Framework for Transmission of IP Datagrams over MPEG-2 Networks", RFC 4259, November 2005.

[SATLABS] SatLabs System Recommendations. Available at www.satlabs.org.

# Authors' Addresses

Stephane Combes

ESTEC

European Space Agency

Keplerlaan 1 – P.O. Box 299

2200 AG Noordwijk ZH

The Netherlands

Email: stephane.combes@esa.int

URL:   telecom.esa.int

Petter Chr. Amundsen

VeriSat AS

P.O Box 1

1330 Fornebu

Norway

Email: pca@verisat.no

URL:   www.verisat.no

Micheline Lambert

Advantech Satellite Networks

2341 boul. Alfred-Nobel

Saint-Laurent (Montreal,)H4S 2A9

Quebec, Canada

Email: micheline.lambert@advantechamt.com

URL:   www.advantechsatnet.com

Hans-Peter Lexow

STM Norway

Vollsveien 21

1366 Lysaker

Norway

Email: hlexow@stmi.com

URL:   www.stmi.com

# Intellectual Property Statement

The IETF takes no position regarding the validity or scope of any Intellectual Property Rights or other rights that might be claimed to pertain to the implementation or use of the technology described in this document or the extent to which any license under such rights might or might not be available; nor does it represent that it has made any independent effort to identify any such rights.  Information on the procedures with respect to rights in RFC documents can be found in BCP 78 and BCP 79.

Copies of IPR disclosures made to the IETF Secretariat and any assurances of licenses to be made available, or the result of an attempt made to obtain a general license or permission for the use of such proprietary rights by implementers or users of this specification can be obtained from the IETF on-line IPR repository at http://www.ietf.org/ipr.

The IETF invites any interested party to bring to its attention any copyrights, patents or patent applications, or other proprietary rights that may cover technology that may be required to implement this standard.  Please address the information to the IETF at ietf-ipr@ietf.org.

# Disclaimer of Validity

This document and the information contained herein are provided on an "AS IS" basis and THE CONTRIBUTOR, THE ORGANIZATION HE/SHE REPRESENTS OR IS SPONSORED BY (IF ANY), THE INTERNET SOCIETY, THE IETF TRUST AND THE INTERNET ENGINEERING TASK FORCE DISCLAIM ALL WARRANTIES, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO ANY WARRANTY THAT THE USE OF  THE INFORMATION HEREIN WILL NOT INFRINGE ANY RIGHTS OR ANY IMPLIED WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

# Copyright Statement

Copyright (C) The IETF Trust (2008).

This document is subject to the rights, licenses and restrictions

contained in BCP 78, and except as set forth therein, the authors  retain all their rights.

<<< RFC Ed Note: please remove the text below, prior to publications >>>

Change Log.

Rev -00

* First draft, for comment by the community.

Rev -01

* Second draft, for comment by the community.

The MIB module is renamed DVBRCS-MIB to reflect that it covers more than solely interface issues.

The MIB has been updated with

* a conformance section that captures the options

* structural changes and corrections to achieve this

* SYNTAX refinements where applicable

* UNITS declarations where applicable

Document reformatted to conform to I-D format conventions.

Rev -02

* Third draft, for comment by the community.

Update following comments received from the ipdvb list (Gorry Fairhurst).

Rev -03

* Fourth draft, for comment by the community.

Update following comments received from Martin Stiemerling and complement/corrections from the SatLabs Group.

Rev -04

* Fifth draft

Corrected read-write into read-create MAXACCESS for objects in requestClassTable and pidPoolTable.

Minor updates to correct format of tables and some MIB comments.

<<<End of RFC Ed Note >>